Evaluation of haptic training: application description

Dan Morris, 4/11/2006

This application demonstrates some of the features that we’ve discussed for our “haptic mentoring” experiments, including random trajectory generation and several forms of haptic feedback.

There are three sections to this document:

- Interface

- Running a study

- Internal details

- Methods

Interface

To get started, you should just run the “haptic_training.exe” program in the path where the included dll’s live (i.e. run it from the path you unzip to).  You will see the “ground plane” in red, with a randomly-generated trajectory painted on it.  If you want a new trajectory, you can press “generate trajectory” at any time, as long as the haptic device is currently disabled (the “generate trajectory” function is disabled when the haptic device is enabled).

GL window

You’ll also see two spheres:

- The red sphere represents the position of your haptic device

- The blue sphere represents the current animation position when the trajectory is being animated (I’ll discuss this more later), so it only moves when you press ‘play’.

- The purple sphere is the point on the trajectory that’s closest to your haptic device position (computed using a kd-tree-based nearest-neighbor query).

When you’re ready, press “enable haptics” to turn on the haptic device.  You can move the device around and your nearest position on the curve will be tracked.  You can now turn on one or more of the forces controlled by the checkboxes (make sure to hold on to your device; I’ve tested this with no problems on my omni, but you know how moving a haptic application to a new machine can be a little unpredictable).

Checkboxes

- ‘plane proxy’ just makes the plane a physical object using the standard CHAI proxy.

- ‘plane constraint’ constrains your device to lie on the “ground plane”.  This will be very useful as a test mode; in this mode, we can let the user move along the curve and evaluate how hard they’re pressing in the normal direction (in free-motion mode, without some opposing force, it’s impossible to measure how hard they’re pushing).

- ‘curve constraint’ pulls your device toward the nearest point on the curve.  The best analogy here is that it should feel like the children’s toy in which you slide a bead along a wire.  This may be useful for the same reason as the “plane constraint” mode.

- ‘guidance’ lets the system “pull you along” the trajectory.  If you enable “guidance” and press “play”, you’ll see the blue sphere start moving and you’ll be dragged along with it.  We have not discussed in detail whether there would be a particular time component to our trajectories, or whether the user would move freely along the curve.  Having the trajectory run in a fixed time certainly simplifies the analysis somewhat.

Velocity can vary sinusoidally along the curve, but is currently set to be constant.  Also FYI you can press “play” when haptics are disabled to just see the animation.

- ‘normal force’ enables the rendering of the plane-normal forces that are embedded in the trajectory.  These forces are currently displayed visually as the width of the line.  A thin line is a zero force, a thick line is a positive force.  We have the ability to add negative forces too.  I was limited to a very small range of line widths for my first implementation pass; we can fix that later too.

If you have ‘guidance’ disabled, so there’s no animation, and you enable ‘normal force’ the normal force comes from the “purple point”: the closest point on the curve to your device.  With no other constraints, this is a very difficult mode to use the program in, since you can introduce large force discontinuities.

Sliders

Each of these forces – plane constraint, curve constraint, guidance constraint, and normal forces – is associated with a force constant, which can be controlled from a slider on the right.  Additional sliders control the maximum allowable force (N) and the level of velocity damping (N / (chai_units / s)).

Graphical force indicator

Checking ‘show target force’ or ‘show applied force’ brings up a visual indicator of the current applied force (in green) and/or the current ‘target’ level of force (in blue).  This is intended as a visual training mode, where one might have ‘guidance’ enabled and ‘plane constraint’ enabled, but ‘normal force’ disabled.  Your goal would be to line up the green bar with the blue bar.

The visual indicator travels with the “guide position” so you can focus your attention in one horizontal location.

Running a trajectory

Press ‘play’ to start a trajectory.  If guidance is enabled, a message will appear asking you to move your device to the start position, and you will be gently guided in that direction. This is to avoid initial force transients and to make sure that the user starts a trajectory in the right place.  When you get there, the trajectory will begin.

When the trajectory ends, the guidance force will disappear.  If you press ‘play’ again, the trajectory will play backwards.  This is just a development convenience and we won’t use it in our study.  If you press ‘reset’, the trajectory will start over.

Running a study

[more detailed study instructions are listed in haptic_mentoring_instructions.doc]

Pressing “start experiment” will automatically do the following:

- Enable haptics, if they’re disabled

- Generate a random sequence of training conditions, based on the contents of the haptic_training.ini file

- Run the trials

Internal Details

Here I’ll describe the algorithms used to generate trajectories and the algorithms used to apply forces to the device.

Trajectory generation

There are three stages to generating a trajectory:

- Generate a curve of x/z positions (y is always 0).  Currently the curve is discretized into 2000 linear segments.

- Generate times for each point on that curve, according to a varying set of (optionally varying) velocities

- Generate forces for each point on that curve

Generating curve positions

- 20 sinusoids are generated, each consisting of randomly-selected DC offsets (in z), amplitudes, frequencies, and phase shifts.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a z value.

- The z values are all then normalized to fit into the desired range (i.e. so they fit on the plane that we show on the screen).

- We now compute the total length of the curve by adding the distances between each pair of successive points

- Then we go through each point to compute its arc-position, i.e. a number from 0.0 to 1.0 that says “how far along the curve is this point?”

All the (x,y,z) points are then stored in a kd-tree for rapid nearest-neighbor queries as we move the haptic probe around later.  The kd-tree is constructed and queried using:

http://www.cs.umd.edu/~mount/ANN/

Generating curve times

In “constant velocity mode” :

- The velocity is constant at 0.6 chai_units / s

- Assign time 0.0 to point 0

- Step along the curve, using the (already computed) arc-distance between each successive pair of points and the constant velocity to assign a time to each point

In “variable velocity mode” (currently disabled) :

- 3 sinusoids are generated, each consisting of randomly-selected DC offsets, amplitudes, frequencies, and phase shifts.  These sinusoids will represent velocity at each point.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a velocity value.

- The velocity values are all then normalized to fit into the desired range.

- We then go through each point and compute the time at which we should reach that point, using the velocity we computed at each point and the distance between each point and its predecessor.

Generating curve forces

- 4 sinusoids are generated, each consisting of randomly-selected DC offsets, amplitudes, frequencies, and phase shifts.  These sinusoids will represent normal force at each point.

- We step along the x direction, computing the value of each sinusoid and adding them up to get a force value.

- The force values are all then normalized to fit into the desired range.

Force computation

The forces corresponding to each ‘check box’ are computed independently, then added up to give a net force.  If the overall force magnitude is greater than the maximum allowable force, it saturates (i.e. it is scaled down to preserve its direction).

Plane proxy

This force is generated from the standard CHAI proxy, using the plane as a mesh.

Plane constraint

A penalty force is generated that tends to move the users haptic probe toward the y=0 plane.  In other words,

fplane constraint = (0,fy,0)

fy = kplane constraint * (0 – device_position.y)

kplane constraint is the value on the GUI’s slider, defined in N / chai_unit .

Normal force

The “current” normal force is looked up in the vector of forces that we computed from random sinusoids, and applied directly to the device.

If we’re playing a trajectory (the usual situation when you’d want to apply the recorded normal force), the “current” normal force comes from the “current” position along the trajectory, regardless of where the user’s probe is.

If the user is moving around but no trajectory is being played, the “current” normal force comes from the closest point on the curve to the user’s probe.  If this is confusing, ignore it; we will almost never use this mode.

Curve constraint

This force constrains the user to move toward the curve.  If dp is the current device position and cp is the closest point on the curve to dp, the force is:

fcurve = kcurve * (cp – dp)

kcurve is defined on the GUI’s slider, and is defined in N/chai_unit .

The closest point on the curve to the current haptic device position is computed as follows:

- Call the user’s probe position P

- All points on the curve (2000 points per curve in this application) are stored in a 3-dimensional kd-tree, along with indexing information that stores the topology of the curve (i.e. they’re not just stored as a point cloud).  Each point is associated with an index that says where it lies on the curve; e.g. the first point is 0 and the last point is 1999.  The notation pi refers to the point with index i.

- At each timestep, we want to find the closest point in the curve to P, so we:

  - Ask the kd-tree for the N points that are closest to P.  I use N=6; higher N cost more time, and lower N can potentially create closest-point errors.  Why not just ask for the ONE closest point to P?  See the figure below for an example.  Here the blue circle represents ‘P’ (the haptic device), and each numbered point is a sample on the curve.  The haptic device should clearly be constrained to the segment between points 1 and 2, but in fact the closest point is point 4.  This is why we have N != 1.

  - For each of the N nearest points pi:

    - Compute the closest point to P on the line segment between pi and pi-1.  In the graphical example below, we would first have retrieved point 4, so we would compute the closet point between P (the blue dot) and the line segments 34 and 45.

    - If either of these line segments store the best closest-point yet, store it.

  - After performing a total of 2N closest-point-on-line-segment computations, we take the best closest-point that we found and call that the current position along the curve.

Guidance force

This force constraints the user to move with the current position in a playing trajectory, i.e. the “guidance ball”.  This force only moves the user in the xz (tangential) plane.  If dp is the current device position and gp is the current guidance position, the force is computed as:

fguidance = kguidance * (gp – dp)

fguidance.y = 0;

kguidance is defined on the GUI’s slider, and is defined in N/chai_unit .

Velocity damping

With all these penalty forces, instability is a real issue in some cases, so we apply a straightforward velocity damping to reduce instability.  Velocity is computed over a 30-millisecond window and a force is applied to oppose the velocity.  If the current velocity is v, the damping force is computed as:

fdamping = -1.0 * kdamping * v

kdamping is defined on the GUI’s slider, and is defined in N/chai_unit .  By default, the magnitude of this force is quite small (imperceptible at “typical” velocities) but has a significant affect on stability.